Tech-ni-Fold Micro-Perforator Tool Kiss-Cut Attachment

Converting your Tech-ni-Fold Micro-Perforator to a Kiss-Cutting Device is quick and easy.

Kiss-Cutting both self-adhesive label stock and thicker card stocks is now possible on your folding machine.

Kiss-Cutting on card stock

Kiss-Cutting on self-adhesive label stock

Instructions

- Setup the Micro-Perforator Blade Holder as you would for a Micro-Perforation job, but instead replace with the supplied Kiss-Cutting Blade

Please take extra care removing the Kiss-Cutting Blade from the packaging.

The Blades are extremely sharp.

- Install the Black Nylon Sleeve to the Female Sleeve Holder component

- Install both the Blade Holder Component and the Nylon Holder Component to the shafts of the machine as per normal installation procedure

- Set the usual shaft pressures according to the stock you are processing through the machine and run a sheet through

- Check the kiss-cut (half-cut) result achieved on the sheet is as required

- Adjust the pressures on the machine to increase or decrease cut penetration

For greater cut depth then experiment with

replacing the Blue Nylon Sleeve for the Black Nylon Sleeve.