Installation of Muller Panda 2/ Tigra

Code QC-MU/PA-FP-6-01

30mm shafts with 80mm outer diameter creasing unit

Top shaft                                                      Bottom shaft

Top shaft                                                       Bottom shaft

Uses a single wedge shaped crease rib                Uses a double wedge shaped crease rib

NYLON CREASE RIBS

Remove the crease ribs from crease units before sliding on to the shafts  as accidental damage to the crease ribs can occur during installation.

3 depths of crease can be obtained by using the 3 colour coded crease ribs as shown below. Each rib has a coloured dot on the inside of the crease rib.

orange             blue            yellow

light            medium        heavy

2 x Top Shaft Support Collars

Bottom shaft crease unit

Top shaft crease unit

Aligning the Creasing units

With the Nylon crease ribs locked into the crease units, align the BOTTOM SHAFT crease units to the correct creasing position   ( left side being aligned with the edge of the clamp ) and tighten the fixing screws

Slowly lower the top shaft crease units making sure the top nylon crease ribs locate into the female channel on the bottom crease unit.

DO NOT TIGHTEN TOP SHAFT FIXING SCREWS

Slowly run a sheet of cover stock through the Spine & Hinge units.

As the cover travels through the crease units the loose top crease units will automatically centralise to the optimum creasing position.

With the cover stock still gripped between the crease units tighten the fixing screws.

The machine is now ready to test.